Literature Review – E Demand of Seasonings/ Additives

Category 1 - Energy Demand of Hard Spices

Categories

- Hard spices - Pepper

- Additives – Mustard

Part 1.1 - Pepper Production Literature12

- Raw Sweet red pepper

- For 1kg of final pepper paste – final solid mass (dry matter content) ~34% w/w (Brix) = 0.34 kg (Table 2) (34% solids, 66% water)

- Total mass content - 7% solids (fresh pepper containing mass) (66% water)

Fresh pepper mass mfresh= final solid masstotal solid content= 0.34 kg0,07=4.857 kg required to make a 1kg thick paste

- Fresh water = Fresh pepper – solid pepper = 5.36 – 0.34 = 5.02 kg

- Step 1: Pretreatment

- Water required for washing pepper mw = 4.857 ltr = 4.857 kg

- Fresh water = 4.857 - 0.34 = 4.517 kg

- Final water = 1 - 0.34 = 0.66 kg

- Water to evaporate = 4.517 - 0.66 = 3.857 kg

- ΔT = Twash​ - Tinlet ​= 40 - 20 = 20 K Literature13 (Drying T at 40 C, 20 C as Ambient T)

- Energy for washing  Q=mfresh* Cp* ΔT = 4.857 kg * 4.186 KJ/kg K * 20 K = 406.7 kJ = 406.7 /3600 = 0.113 kWh

- Pump Energy Epump = 33.5 kW * 1h / 3000 kg = 0.0112 kWh (Table 3 - Literature12)

- Energy demand of pretreatment Epretreatement= 0.11+0.05=0.16kWhkg

- Step 2: Pepper Crushing

- Pepper Crushing Machine Ecrushing =4 kW*1h2500 kg=0.002kWhkg

- Step 3: Preheating (85 C/1-2 min)

- Preheat from 85 – 20 C = ΔTpreheat = 65 K (assuming Ambient to be 20 C)

- mfresh=4.857 kg

- Thermal efficiency incl. transfer losses η = 0.80 (assumption)

- Q=mfresh* Cp* ΔT = 4.857 kg * 4.186 kJ/kg K * 65 K = 1,321 kJ = 0.367 kWh

- Energy demand of preheating Epreheat=Qpreheatη =0.3760.8=0.46kWhkg

- Step 4: Pepper Milling

- Pepper milling Machine Emiling=37 kW * 1h3500 kg =0.01kWhkg

(STEPS 6-13: NOT INCLUDED IN THE MODEL)

- First Evaporation (Table 3)

- Steam consumption: 500 kg/h; Evaporating capacity: 1500 kg/h; Capacity: 2500 kg/h; Pel= 22.5 kW

- Steam at atmospheric conditions (1 bar ≈ 100 °C) (approximation)

hsteam=2257kJkg=22573600=0.627kWhkg

- Thermal Energy of Steam Esteam=500kgh*0.627kWhkg=313.5 kW

- Specific Energy Eper kg feed=313.5 kW2500kgh=0.125kWhkg feed

- Electrical Energy of Evaporator Eelectric=22.5 kW2500 kg/h=0.009kWhkg feed

- Energy demand of Evaporator per kg feed

Efirst evaporation=0.125+0.009=0.134kWhkg

(OR)

- Salt addition

  - Average CEnC factory processing = (table 6)

1266+960+1266+1266+960+960MJt6 =1,113MJt=0.309kWhkg

  - 3% Salt Addition = 0.03* 0.309=0.0093kWhkg

  - Esalt addition=0.0093kWhkg

- Second Evaporation – multi effect (half the E per kg, apart of latent heat is reused)

  - Eseconf evaporation=0.1342=0.067kWhkg

- Pasteurization (90 – 95 C)

  - ΔT=95-20=75 K

  - Qpastuerize=m* Cp* ΔT=1 kg*4.186kJkg K*75K=313.95 kJ3600 s=0.087 kWh

  - Epastueriation=Qpastuerizeη=0.0870.8=0.108kWhkg

- Filling (Aseptic/ normal)

  - Efilling, total= Eelectric, total per kg+ Esteam, total per kg

  - Factory Capacity = 2500 kg/h

  - Operating time: t per 1 tonne = 1000 kg/ 2500 kg/h = 0.4 h

  - Etank,filling=  P*t=1.5 kW*0.4 h=0.60 kWh/t

  - Aseptic filler time taseptic=186.9 kg3000kgh=0.0623 h

  - Balance tank time ttank=186.9 kg1400kgh=0.1335 h

  - Etank,aseptic=2.2 kW* 0.0623 h= 0.1371 kWh

  - Etank,tank=1.5 kW*0.1335 h=0.2002 kWh

  - Eelectric, total=0.1371+0.2002= 0.3373 kWh

  - Eelectric, total per kg=0.33730.3373 kWh186.9 kg=0.001805kWhkg

  - msteam=mflow*t=40kgh*0.0623 h=2.492 kg

  - Esteam =msteam*Hvap=2.492 kg*2257kJkg3600 =1.561 kWh

  - Esteam, total per kg=1.561 kWh186.9 kg=0.00835kWhkg

  - Efilling=0.00835+0.001805=0.01016 kWhkg

- Lidding/ Labelling

  - Lidding: ELidding= 3.25 kg  * 9.30 MJ/kg=30.225 MJ/ batch

  - Labeling: ELabelng=3.58 kg  * 83.8 MJ/kg = 299.984 MJ/ batch

  - Total: ETotal=30.225+299.984 = 330.209 MJ/ batch

  - Specific Energy Especific= 330.209 MJ/batch186.9 kg paste batch * 3.6 MJkWh= 0.491kWhkg paste

- Cooling (Tubuler sterlization)

  - Sterilizer operating time for the batch: t=186.9 kg500kgh=0.374 h

  - Cooling water mass used during the batch:

mcw=3000kgh×0.3738 h=1121 kg

  - Thermal energy removed by that cooling water (assuming ΔT = 5 °C):

Ecool per batch=mcw×cp×ΔT=1121 kg×4.186 kJ/kgK×5 K=23483 kJ=23.483 MJ3.6=6.523 kWh per batch

Ecooling=6.523 kWh186.9 kg=0.0349kWhkg

- Carton boxing (Case tipping machine)

  - Operating time: tboxing=186.9 kg3000kgh=0.0623 h

  - Electrical energy consumed by the boxing machine (per batch):

Eboxing,batch=11 kW×0.0623 h=0.685 kWh

  - Eboxing=0.685 kWh/186.9 kg=0.0036kWhkg

- TOTAL

  - Eprocess= Epretreatement+ Ecrushing+ Epreheat+ Emilling+ Efirst evaporation+ Esalt addition+ Esecond evaporation+ Epastueriation+Efilling+ Elidding+ Elabeling+Ecooling+Eboxing

  - Eprocess= 0.162+ 0.0016+ 0.459+0.0106+0.134+0.0093+ 0.067+0.108+0.01016+0.491+0.0349+ 0.0036 =  1.57326 kWh/kg

  - Old value came up to 4.01316kWhkg, modified Epreheat and Eglass &  EPET from 1.532 & 1.48

From Literature1, table 6:

Ewhole process= 7146+5385+7372+5959+4197+4118MJt6=5696.17

Ewhole process=5696.17MJt1000kgt*3.6MJkWh=1.582kWhkg

Pepper Powder Steps: Literature14

Assumptions stay the same as before (only TS changed):

  - Latent heat of vaporisation: hfg= 2257kJ/kg

  - Dryer efficiency: ηd=0.80

  - Electrical use for fans etc.: 0.1 kWh/kg water evaporated

- Step 5: Drying/Dehydration (Per kg paste)

Thermal part:

Qth=mevap⋅hfgηd=0.622⋅22570.8=1755 kJ⇒1755/3600≈0.487 kWh/kg paste

Electrical part:

Eel=0.1 kWhkg H₂O⋅0.622=0.062 kWh/kg paste

E5, paste=0.5+0.06=0.56 kWh/kg paste

- Step 6: Grinding

- Pepper Mill: 3.6 kW, 260 kg/h  = 0.072 kWh/kg

E6, powder=0.03 kWh/kg powder

- TOTAL

  - Eprocess= Epretreatement+ Ecrushing+ Epreheat+ Emilling+Edrying+Emilling

  - Eprocess= 0.16+ 0.002+ 0.46+0.01+0.56+0.03= 1.22 kWh/kg

  - Conversion factor=2.65 (Convert paste to powder)

  - Epepper powder = 3.22 kWh/kg powder

Pepper Powder – Comparison Literature 15

  - SEC Range: 3.13 – 270.87 kWh/kg H₂O

  - mwater = 1.65 kg H₂O/kg powder

  - Epepper powder = 5.16 kWh/kg powder

Category 2 - Energy Demand of Additives

Part 2.1 - Mustard production Literature 18

- Mustard seed feeding (conveyors / feeders)

- Cracker

- Kolhu division

- Produces Kacchi Ghani oil + oil cake

- Double pass expeller

- Single pass expeller

- Produces Pakki Ghani oil + final oil cake

- Filtration

- Packing

Feed (mustard seed) into expeller line: 1600 kg/hr

- 1.6 tonnes/hr = 16 quintals/hr
(1 quintal = 100 kg)

- Mustard Seed Feeding - Auxiliary power - Literature 19

PSeed feeding= 1.5 kW

ESeed feeding=Pactual (kW)Throughput (quintalhr)=1.516=0.0937kWhquintal

- Seed Cracking – Literature 20

PCracking rated= 15 HP=11 kW  (pg. ix)

ECracking=Pactual (kW)Throughput (quintalhr)=1116=0.6875kWhquintal

- Kolhu Division – Literature 21

PKolhu Division=40 / 50 HP=30/ 56 kW (new) (pg. 5)

EKolhu Division=Pactual (kW)Throughput quintalhr=30 or 5616=1.875 or 3.5kWhquintal

- Double Pass Expeller - Literature 18

PDouble pass expeller = 60 HP=40 kW (old) (pg. ix)

EDouble pass expeller=Pactual (kW)Throughput (quintalhr)=4016=2.5kWhquintal

- Single Pass Expeller - Literature 18

PSingle pass expeller =50 HP= 32.5 kW (new) (pg. ix)

ESingle pass expeller =Pactual (kW)Throughput (quintalhr)=32.516=2.031kWhquintal

- Filtration – Literature 22

PFiltration = 7.5 kW

EFiltration=Pactual (kW)Throughput (quintalhr)=7.516=0.4687kWhquintal

- Packing - https://www.shivmachinetech.com/edible-oil-filling-machine.html

PPacking rated = 1.5 kW(15 kg Mustard Oil filling machine)

EPacking=Pactual (kW)Throughput (quintalhr)=1.516=0.0937kWhquintal

- Total

Eprocess= ESeed feeding+ ECracking+EKolhu Division+EDouble pass expeller+ESingle pass expeller + EFiltration

Eprocess=0.09+0.69+3.5+2.5+2.03+0.47=9.28 kWh/quintal

Hence, assuming PKolhu division, rated =56 kW, Eprocess= 9.3746 kWh/Quintal

From Literature 18, pg. 1:

EBEE = 10-15 kWh/Quintal

Mustard Oil – Comparison – KVIC Literature 24

  - Production capacity = 6600 ltr Oil

  - Electricity Consumption = 12000 kWh/year

  - mseed=198 quintals/year

  - SECmustard oil=50.5 kWh/quintal